Lora Alarm System Application Communication Protocol

V1.4

版本历史

| Ver | Date | Revision Author | modify content |
| --- | --- | --- | --- |
| 1.0 | 2026-5-6 | Mr G | First version |
| 1.1 | 2026-5-11 | Mr G | Add SRP related support |
| 1.2 |  |  | Incorporate the feedback received |
| 1.3 | 2026-5-19 | Mr G | Modify CMD 0x02,ADD latitude and longitude |
| 1.4 | 2026-5-21 | Mr G | Modify the Code and group id related protocol |

蓝色字体表示新增

紫色字体表示修改

灰色表示未实现

目录

Lora Alarm System Application Communication Protocol	1

1 communication interface	4

1.1 Upstream and downstream port descriptions	4

1.2 Other Features	4

2 Instruction Details	4

2.1 instruction format	4

2.3Instruction Details	7

3. CRC16	12

- communication interface

  - Upstream and downstream port descriptions

This protocol is used for badge, hub and LNS.

  - Other Features

2 Instruction Details

2.1 instruction format

| head | ver | control | CMDID | length | CRC16 | data |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 2 | 2 | N |
| 0xAA55 | X | X | X | XX | XX | X… |

head

The length is 2 bytes, fixed to 0xAA55

ver

Length: 1 byte, default is 1

CMDID

| CMDID | CMD NAME | Instruction Description |
| --- | --- | --- |
| 0x00 | heartbeat | The gateway and hub report a heartbeat every 5 minutes. |
| 0x01 | power | The badge and hub report any change in battery level when the interval exceeds 5 minutes. |
| 0x02 | Key Event | After the button is triggered, report the button event. |
| 0x03 | Code | Set operation codes (red code, blue code, yellow code，green code，Hold Code, Secure Code, Evacuate Code, Shelter Code) |
| 0x04 | Code Setting | Set the display content for the code. |
| 0x05 | Led control | Control the led |
| 0x06 | Buzzer control | Control the buzzer |
| 0x07 | Vibration control | Control the vibration |
| 0x08 | LCD context control | LINE1 & LINE2 Content Control |
| 0x09 | LCD LINE2 onoff | Decision whether to display the second line of text |
| 0x0A | Clear Packet | Clear all or all clear the status |
| 0x50 | Set group id | Set device group id |

control

The length is 1 byte, defined as follows:

| Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RESERVE |  |  |  | RESEND | PACKET_TYPE |  |  |

PACKET_TYPE：

0 -- Request packet; others reserved

RESEND：0

RESERVE：0

length

Includes all header information and the length of data fields

CRC16

The CRC16 check values for both the header (excluding the CRC16 field) and data fields are calculated using the CRC16/XMODEM verification algorithm.

Data Field

See the specific format of each instruction code.

2.3Instruction Details

CMDID 0x01~0x02 for uplink

Other CMDID for downlink

2.3.1 command list (CLIST)

2.3.1.1 heartbeat（0x00）

Direction: badge/hub -> Gateway

Instruction: The device reports a heartbeat packet every 5 minutes.

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| device type | 1 | 0 or 1 | 0 for badge 1 for hub |
| Group id | 1 | 0~255 | Default is 0, 0 means no role. It is set by CMDID 0x50. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: (reserve) |

| Example | (head) （ver） （control）（CMDID）（length）（CRC16）（data） AA55 01 00 01 0A 00 xx xx 01 |
| --- | --- |

2.3.1.2 power（0x01）

Direction: badge/hub -> Gateway

Instruction: Report a change in battery level once, with an interval of no less than 5 minutes.

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| device type | 1 | 0 or 1 | 0 for badge 1 for hub |
| power | 1 | 0~100 | Battery precent |

2.3.1.3 Key Event（0x02）

Direction: badge/hub -> Gateway

Instruction: Report when the button is triggered

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Button | 1 | 0~3 | 0 for green, 1 for blue, 2 for yellow , 3 for red (key) |
| motion | 1 | 0 or 1 | 0 indicates a short press 1 indicates a long press |
| rssi | 1 | 0~255 | Indicates Bluetooth signal strength. -90 dBm converts to 90 |
| Hub mac | 6 | XX:XX:XX:XX:XX:XX | Hub mac addr |
| latitude | 4 | Bit31: 0 N,1S Bit0~30: 0~90000001 | 90000000 means 90.000000 90000001 means invaild |
| longitude | 4 | Bit31: 0 E,1W Bit0~30: 0~180000001 | 180000000 means 180.000000 180000001 means invaild |

2.3.1.4 Code（0x03）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| Alarm | 1 | 1~7 | 0 for green, 1 for blue, 2 for yellow , 3 for red,4~7 for Hold, Secure, Evacuate, Shelter alerts |

2.3.1.5 Code Setting（0x04）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| Alarm | 1 | 0~7 | 0 for green, 1 for blue, 2 for yellow , 3 for red,4~7 for Hold, Secure, Evacuate, Shelter alerts |
| Enable switch | 1 | 0x00~0xFF | Bit0: lcd line 2 enable Bit1: buzzer enable Bit2: vibration enable |
| Mode Switch | 1 | 0x00~0xFF | Bit0: buzzer mode, 0 for normal, 1 for twinkle. Bit1: vibration mode |
| Volume | 1 | 0~10 | 0 is the minimum volume, and 10 is the maximum volume. |
| Buzzer Open duration | 4 | 0~2147483647 | Unit ms |
| Buzzer CLose duration | 4 | 0~2147483647 | Unit ms |
| VibrationOpen duration | 4 | 0~2147483647 | Unit ms |
| VibrationCLose duration | 4 | 0~2147483647 | Unit ms |
| LED_R | 1 | 0~255 |  |
| LED_G | 1 | 0~255 |  |
| LED_B | 1 | 0~255 |  |

2.3.1.6 Led control（0x05）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| Color | 1 | 0~3 | 0 for green, 1 for blue, 2 for yellow , 3 for red,4~7 for Hold, Secure, Evacuate, Shelter alerts |
| Mode Switch | 1 | 0x00~0xFF | Bit0: led mode, 0 for normal, 1 for twinkle. |
| Onoff | 1 | 0 or 1 | when mode is 0, 0 if off, 1 is on . When mode is 1, this byte is ignore. |
| Led Open duration | 4 | 0~2147483647 | Unit ms |
| Led CLose duration | 4 | 0~2147483647 | Unit ms |
| LED_R | 1 | 0~255 | Color precent |
| LED_G | 1 | 0~255 | Color precent |
| LED_B | 1 | 0~255 | Color precent |

2.3.1.7 Buzzer control（0x06）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| Mode Switch | 1 | 0x00~0xFF | Bit0: buzzer mode, 0 for normal, 1 for twinkle. |
| Onoff | 1 | 0 or 1 | when mode is 0, 0 if off, 1 is on . When mode is 1, this byte is ignore. |
| Buzzer Open duration | 4 | 0~2147483647 | Unit ms |
| Buzzer CLose duration | 4 | 0~2147483647 | Unit ms |
| Volume | 1 | 0~10 | 0 is the minimum volume, and 10 is the maximum volume. |

2.3.1.8 Vibration control（0x07）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| Mode Switch | 1 | 0x00~0xFF | Bit0: buzzer mode, 0 for normal, 1 for twinkle. |
| Onoff | 1 | 0 or 1 | when mode is 0, 0 if off, 1 is on . When mode is 1, this byte is ignore. |
| motion | 1 | 0 or 1 | 0 is action, 1 is setting |
| VibrationOpen duration | 4 | 0~2147483647 | Unit ms |
| VibrationCLose duration | 4 | 0~2147483647 | Unit ms |

2.3.1.9 LCD context control（0x08）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| First line content | 20 | U8 str[20] = “Code Red Alert” | The content displayed in the first line |
| Second line content | 20 | U8 str[20] = “Room 4” | The content displayed in the second line |

2.3.1.10 LCD LINE2 onoff（0x09）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| Line 2 enbale | 1 | 0 or 1 | 0 for disable, 1 for enable. If disable, the lcd will not display the line 2 context. |

2.3.1.11 Clear Packet（0x0A）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Means the target role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: all role |
| Clear type | 1 | 0 or 1 | 0 for Clear All Statuses --> all led close 1 for All clear --> green led twinkle |

2.3.1.11 Set group id（0x50）

Direction: Gateway -> badge/hub

Instruction: Cloud-initiated notification

| data division |  |  |  |
| --- | --- | --- | --- |
| field | length （Byte） | data | data specification |
| Group id | 1 | 0~255 | Set the device role. Bit0: admin Bit1：Nurses Bit2：secure Bit3：principal Bit4：......xxx .... Bit7: ( reserve ) |

3. CRC16

CRC verification employs the CRC16/XMODEM checksum code. The C language algorithm code is as follows:

uint16_t xmodem_val;

void crc16_xmodem_init( void )

{

xmodem_val = 0;

}

void crc16_xmodem_append( uint8_t *pbuf, uint16_t length )

{

while( length-- ) {

xmodem_val ^= (uint16_t)(*pbuf++) << 8;

for( uint8_t i = 0; i < 8; ++i )

{

if( xmodem_val & 0x8000 ) {

xmodem_val = (xmodem_val << 1) ^ 0x1021;

} else {

xmodem_val <<= 1;

}

}

}

}

uint16_t crc16_xmodem_end( void )

{

return xmodem_val;

}

Usage Notes:

Before calculating CRC, you must first call crc16_xmodem_init(). For computing CRC16 values from multiple data segments, call init only once. Data requiring verification is appended using crc16_xmodem_append(), and the verification result can be retrieved via crc16_xmodem_end(). Note: Both interfaces can be called multiple times.

Demo:

uint8_t crc_data[] = {

0xAA, 0x55, 0x01, 0x00, 0x00, 0x08, 0x00, 0x03

};

crc16_xmodem_init();

crc16_xmodem_append( crc_data, sizeof(crc_data) );

LOG( "crc16_xmodem value1: %X\r\n", crc16_xmodem_end() );

crc16_xmodem_append( crc_data, sizeof(crc_data) );

LOG( "crc16_xmodem value2: %X\r\n", crc16_xmodem_end() );

Run result：

crc16_xmodem value1: 58C7

crc16_xmodem value2: E228

LED Colors

We need to make sure that the platform supports RBG values instead of defining the colors in a different way. This will enable us to satisfy the requirements for Standard Response protocol (SRP), by sending below colors directly from the dashboard (backend). These are not initiated from a badge:

| Command | Trigger Source | Broadcast Target | LED Color | Badge LCD Message |
| --- | --- | --- | --- | --- |
| Hold | Dashboard only | All Badges & Hubs | Purple | Hold Alert |
| Secure | Dashboard only | All Badges & Hubs | Blue | Secure Alert |
| Evacuate | Dashboard only | All Badges & Hubs | Green | Evacuate Alert |
| Shelter | Dashboard only | All Badges & Hubs | Orange | Shelter Alert |

Below is the table with proposed values

| Alert Type | Color | RGB | Visibility Reason |
| --- | --- | --- | --- |
| Lockdown / Code Red | Red | 255, 0, 0 | Maximum urgency, universally recognized |
| Medical / Secure | Blue | 0, 100, 255 | Distinct from red and visible through diffusers |
| Admin Assist | Yellow | 255, 220, 0 | Very bright, easily seen in daylight |
| All Clear / Evacuate | Green | 0, 255, 80 | Bright but clearly different from yellow |
| Hold | Purple | 180, 0, 255 | Unique color not confused with others |
| Shelter | Orange | 255, 120, 0 | Strong color between red and yellow |

LoraWAN Class (A or B)

You selected Class B indicating that “badge and hub need to rec the downlink of the server”. Our main concern is the battery consumption, particularly for the Hub. Have you considered this?

Offline Fallback

As we continue to review the system design, we wanted to raise a question: how does the system handle an internet outage?

Given that this is an emergency alert system, it is essential that alerts continue to function at the campus level even if all internet connectivity is lost — Ethernet, Wi-Fi, and LTE/4G simultaneously.

Has this been considered and addressed in the current design? Specifically, we would like to understand what the gateway does in the following scenario:

1. All internet connectivity is lost.

2. A badge panic button is pressed on campus.

Our expectation is that the gateway should:

1. Detect the connectivity loss and activate a local fallback mode automatically — no manual intervention required.

2. Switch to an embedded local LNS to continue processing uplink alerts from badges and hubs.

3. Apply alert routing rules locally and issue the correct downlink commands to devices — buzzers, LED colours, and LCD messages.

4. Provide a minimal alert dashboard to on-site staff over the local network.

5. Queue all events locally and sync them to the AWS backend once connectivity is restored, preserving original timestamps for the audit trail.

6. Switch back to the primary AWS LNS automatically on reconnection.

Could you please confirm how this is addressed in the current design, or flag if anything above needs further discussion?