Additional LED Colors – Standard Response Protocol (SRP)

Currently the system support below colors:

- Red

- Blue

- Yellow

- Green

We need to make sure that the platform supports RBG values instead of defining the colors in a different way. This will enable us to satisfy the requirements for Standard Response protocol (SRP), by sending below colors directly from the dashboard (backend). These are not initiated from a badge:

| Command | Trigger Source | Broadcast Target | LED Color | Badge LCD Message |
| --- | --- | --- | --- | --- |
| Hold | Dashboard only | All Badges & Hubs | Purple | Hold Alert |
| Secure | Dashboard only | All Badges & Hubs | Blue | Secure Alert |
| Evacuate | Dashboard only | All Badges & Hubs | Green | Evacuate Alert |
| Shelter | Dashboard only | All Badges & Hubs | Orange | Shelter Alert |

Make sure the LEDs can support below, making the colors on badges and Hubs (in particular) be unmistakable from a distance (better visibility) – values are slightly tuned from the typical 255 combinations:

| Color | R | G | B | Reason |
| --- | --- | --- | --- | --- |
| Red | 255 | 0 | 0 | strong alert |
| Blue | 0 | 80 | 255 | brighter blue |
| Yellow | 255 | 220 | 0 | avoids green tint |
| Green | 0 | 255 | 60 | more visible |
| Purple | 180 | 0 | 255 | richer purple |
| Orange | 255 | 120 | 0 | clearer orange |

Can you propose the Json code that can be sent from the Dashboard for each these colors?

Badge LCD Size

Make the LCD screen to be able to receive a short text message from the dashboard. Eg the school principal sends a broadcast to all staff “first responders in the building”  or to one staff member “post your student status”.