Devices Behavior Summary

Below table summarizes how the devices behave under different scenarios.

Notes:

- Remember that for any badge button-initiated alert, a prompt to confirm must appear on the LCD screen and must be pressed within 4 seconds or alert is not sent. Short press shows battery status on LCD.

- The badge and hub LEDs do not light/illuminate continuously, they flash for defined internal and duration to conserve battery power

- The badge does not vibrate continuously, they vibrate for some defined time and then stop to conserve battery power

- The badge and hub buzzers do not sound continuously.  They buzz for some defined time and then stop to conserve battery power

- For SRP alerts (Initiated from Dashboard), the staff may be required to provide status via their badge, for example:

  - Press Blue for medical assistance

  - Press yellow for missing students

| Command | Trigger Source | Target | LED illumination Color (Badge &Hub) | Buzzer (Badge / Hub) | Vibration (Badge only) | Badge LCD Message |
| --- | --- | --- | --- | --- | --- | --- |
| Lockdown | Red - Button press | All Badges & Hubs | Red | Yes - all | All | Code Red Room XXX |
| Medical | Blue -Button press | Initiating badge and associated DoorHub | Blue for that badge plus admin and nurses badges only, Blue for DoorHub for that room only | Yes – initiating badge, admin & nurses’ badges only. Not Hub | Yes target badges (initiating badge, nurses and admin badges) | Medical Alert Room XXX |
| Admin help | Yellow - Button press | Initiating badge and associated DoorHub | Yellow for that badge plus admin and nurses badges only, Yellow for DoorHub for that room only | Yes – initiating badge, and admin badges only. Not Hub | Yes target badges (initiating and admin badges) | Admin Alert Room XXX |
| All Clear (Room) | Green - Button press | Initiating badge and associated DoorHub | Green for that badge only, Green for DoorHub for that room only | Yes – Badge only | Yes | All Clear |
| Set All Clear Status | Dashboard only | All Badges & Hubs | Green | Yes - all | All | All Clear |
| Clear All Statues | Dashboard only | All Badges & Hubs | None | None | None | None |
| Hold | Dashboard only | All Badges & Hubs | Purple | All badges, Not Hubs | All | Hold Alert |
| Secure | Dashboard only | All Badges & Hubs | Blue | All badges, Not Hubs | All | Secure Alert |
| Evacuate | Dashboard only | All Badges & Hubs | Green | All badges, Not Hubs | All | Evacuate Alert |
| Shelter | Dashboard only | All Badges & Hubs | Orange | All badges, Not Hubs | All | Shelter Alert |
| Reset badge | Long press Yelow+Green | Individual badge | Flash white when resetting | None | Yes | Firmware Version |
| Text | Dashboard only | Individual badges | None | Yes, badge only | Yes | [Short text from admin] |